### Chapter 1 - Basic Computing Concepts

“File-clerk” model of computers:

- A computer shuffles numbers around - writing, reading, erasing, and rewriting different numbers in different locations according to a set of inputs, a set of rules, and a prior history of inputs - and then halts so that the output can be read by humans.

- Read - Modify - Write sequences of numbers

- All computers have at least 3 structures:

  - Storage - holds data

  - Arithmetic Logic Unit - performs operations on data

  - Bus - transports info between storage and ALU

- Basic machine instructions are - load data from RAM, perform operation, save result in RAM

- Relative addressing - instead of using the actual memory address to load/store data - if you know data will be stored in a memory array - you can find the memory address relative to the start of the array

### Chapter 2 - Mechanics of Program Execution

^^ Arithmetic Instruction Format, Memory Access Instruction Format ^^

- Opcode represents a particular instruction - i.e. add, store, load

Programming Model

- Program counter - holds the index for the next instruction inside the program

- Instruction Register - holds the instruction being executed

Basic pattern for executing a program

- Fetch instruction from address in program counter and load it into the instruction register

- Decode the instruction in the register

- Execute the instruction

- Repeat

Clock pulse - clock generator on motherboard creates pulse - each instruction takes 1 pulse

Jump instructions

- Jump to parts of the assembly code based on the memory address

- PSW (Processor Status Word) - register that holds info about the last instruction for conditionals

Booting up - microprocessor is hard-wired to fetch the program to boot up from the RAM

- First program is the BIOS - at the end it jumps to the bootloader program which loads the operating system

### Chapter 3 - Pipelined Execution

Most instructions are abstracted as:

- Fetch

- Decode

- Execute

- Write

Pipelined execution - instead of performing 1 instruction at a time, the CU will work on multiple instructions simultaneously like an assembly line

- The clock is now based on how much we slice the instructions

- Instruction throughput - instructions per clock cycle

  - Average vs maximum theoretical

- Pipeline stall

  - When instructions get stuck - creates bubbles in the pipeline where no instruction is being carried out

- Instruction latency - clock cycles for an instruction to pass through the pipeline

- Pipelining is only as fast as you can subdivide instructions, which has a limit because of physical limitations

- Clock cycles are now 1 per subdivision

### Chapter 4 - Superscalar Execution

Superscalar execution - executing multiple instructions simultaneously

- Now, instructions have to be dispatched while being decoded

- Dispatch - determining if instructions can be executed simultaneously before being sent to separate ALUs

- Modern CPUs send instructions to multiple execution units, not just ALUs

Numbers & computer arithmetic

- 4 different number formats - scalars and vectors of either floating-point or integer

- 2 types of operations

  - Arithmetic - multiplication, addition, division, subtraction etc

  - Logical - AND, OR, XOR, NAND, etc

- Control unit is connected to multiple types of ALUs: Floating-Point Unit, Simple Integer Unit, Complex Integer Unit

  - LSU = Load Store Unit

  - BEU = Branch Execution Unit

ISA (Instruction Set Architecture) - Abstraction built on microprocessor that allows for programs to be cross compatible across different CPUs

Hazards to dispatching

- Data Hazards

  - When the next instruction relies on data from the previous instruction, so they can’t be executed in tandem.

  - EX:

add A, B, C

add C, D, D

  - Forwarding - directly feeding the result into the input port, bypassing the write stage

  - Register renaming - temporarily using extra registers, before moving data back

- Structural Hazards

  - Multiple instructions require the same resource

  - Physical limitations mean that not every ALU can be connected to every register, so we have a register file, which is an array of registers w/ read/write ports

- Control Hazards

  - CPU is at conditional branch and needs to wait to see if the condition is fulfilled

  - Takes time to load the branch from storage - instruction load latency is added

  - Uses branch prediction to circumvent

### Chapter 5 - The Intel Pentium and Pentium Pro